IPK

INTELLIGENT PRECISION COUPLING

WITH INTEGRATED MEASUREMENT UNIT | INDUCTION

|  | PROPERTIES |  |
| --- | --- | --- |
|  | ## FEATURES - Measurement of different dimensions - Measurement error < 1% (torque) - Integrated measurement amplifier - Display of the data on mobile device - Data export as CSV-file - Data transfer via Gateway | ## SPEZIFICATIONS - Bluetooth 5.0 Low Energy - Inductive power supply - Sample rate up to 500 Hz - Recording rate up to 500 Hz - Speed resistance up to 8000 1/min |
|  | ## MEASURED DIMENSIONS - Torque - Rotation speed - Acceleration - Optional: push / pull forces | ## DESIGN - Standard coupling with integrated sensors - Unchanged mechanical properties - Custom design on request |
|  | ## POWER SUPPLY VIA INDUCTION - Permanent power supply - Suitable for permanent measuring tasks and fixed installation in plants and test benches - Condition monitoring of machines and systems in series - Torque control of industrial processes - Quality assurance and monitoring of production processes |  |

| COMPATIBLE COUPLINGS |  |  |  |
| --- | --- | --- | --- |
| LINE-SHAFTS | DISC-PACK-COUPLINGS | SAFETY-COUPLINGS | STIFF COUPLINGS |
| ZA ¦ ZAE ¦ EZ2 | LP2 ¦ LP3 ¦ LP5 ¦ LPH | STF | Flange adapter |

| PROVISION OF MEASURED DATA |  |
| --- | --- |
| ## R+W GATEWAY - Connection on PC / SPS / Cloud solutions - USB-C Port (UART) - 8 analog outputs (-10 to +10 V) - Output of measured values - 8 digital outputs - Status of connection - Signal of occurred events - SMA connector for external antenna ## R+W APP - Display of all measured values - Min- / Max- and average values - Zeroing of values - Different types of diagrams - Detailed progress of measured data - Intuitive gesture control - Recording measurements for analytics e.g. in MS Excel - Data export as CSV-file |  |  |

BUILDUP

The inductive transmission unit consists of an Intelligent Coupling (1), a coil (2) wound around the component, a pick-up (3) and a gateway (4). The pick-up must be positioned close to the rotating coil for uninterrupted power transmission (A). The connection to the gateway is made via two cables (B). The gateway is operated wirelessly via the R+W App (C). The user can connect to the gateway to change settings or display measured values. The antenna (C) should be positioned near the end device (tablet or smartphone). The following functions are provided via the app: - Commissioning of the inductive coupling - Definition of threshold values and configuration of the data logger - Download of stored measurement data - Display and log measured values at up to 500 Hz in parallel with analog output - Change settings (name, calibration values, taring)

| TECHNICAL DATA – INTELLIGENT PRECISION COUPLING |  |
| --- | --- |
| TORQUE MEASUREMENT. Resolution; 24; bit. Sample rate; 1 – 500; Hz. Error of indication; < 1; %. SPEED MEASUREMENT. Resolution; 16; bit. Sample rate; 1 – 500; Hz. Error of indication 0…660 1/min; 0,5; %. Error of indication 660…2500 1/min; 5; %. ACCELERATION MEASUREMENT. Number of axes; 3. Acceleration (max.); 30; g. Resolution; 16; bit. Sample rate; 1; kHz. OPTIONAL PUSH / PULL FORCE MEASUREMENT. Resolution; 24; bit. Sample rate; 1 – 500; Hz. Error of indication; < 2; %. | OPERATION. Max. rotation speed; 8000; 1/min. Ambient conditions; normal. Temperature; -20 – 60 -4 – 140; °C °F. rel. centrifugal acceleration; 130; g. ESD; 4; kV. Protection class; IP 65. OPTIONAL SPEED SENSOR. For higher accuracy or a measuring range above 2500 1/min, an optional speed sensor can be installed. Further details on request.. INCLUDED ACCESSORIES. 1x; Calibration certificate. 1x; Operation manual. |

| TECHNICAL DATA – GATEWAY |  |
| --- | --- |
| The connections for supply and outputs are located on the top and bottom side of the gateway. | POS.; DESCRIPTION. 1; Digital outputs (do). 2; Digital outputs (do). 3; Power in (24 V) / power out (12 V). 4; Not used / debug. 5; Analog outputs (ao). 6; Analog outputs (ao). 7; External antenna connection, signal input. 8; Status LED. 9; USB-C port. 10; External antenna connection, signal output. 11; Fastening for mounting rail. |

| TECHNICAL DATA – GATEWAY (CONTINUATION) |  |
| --- | --- |
| ANALOG OUTPUTS. Amount; 8. Voltage range; ± 10; V. Resolution; 16; bit. Linearity deviation; < 0,1; %. Max. load; 20; mA. DIGITAL OUTPUTS. Amount; 8. Voltage; 24; V. Max. load; 0,5; A. SERIAL OUTPUT (UART). Amount; 1. Baud rate; Adjustable. Stop bits; 1. Parity; None. Data bits; 8. DATA TRANSFER. Frequency; 2,45; GHz. Bluetooth version; 5.0. Latency; < 150; ms. Synchronicity; ± 25; ms. Transmission power(max.); 4; dBm. Reception strength (min.); -90; dBm. Range1; 10; m. 1; Specification valid for unobstructed view. With partial shielding, the range can be significantly reduced.. POWER SUPPLY. Supply voltage; 24; V. Current consumption (typ. / max.); 50 / 100; mA. Power consumption (typ. / max.); 1,2 / 2,4; W. OPERATION. Temperature; -20 – 60 -4 – 140; °C °F. Protection class; IP 20. Mounting rail; DIN EN 50022. | PIN ASSIGNMENT. PIN; POS. 1; POS. 2; POS. 3; POS. 5; POS. 6. 1; do5; do1; n.c.; GND; GND. 2; do6; do2; GND; ao8; ao4. 3; do7; do3; n.c.; ao7; ao3. 4; do8; do4; 24 V; ao6; ao2. 5; GND; GND; -; ao5; ao1. DIMENSION OF HOUSING. Weight: approx. 150 g. INCLUDED ACCESSORIES. 1x; Cable pick-up – Gateway 1,5 m. 2x; Antenna extension cable 1,5 m. 1x; Antenna (Laird, 001-0001). 1x; Connector, 4-pin (Amphenol, 20020006-G041B01LF). 4x; Connector, 5-pin (Amphenol, TJ0511530000G). OPTIONAL ACCESSORIES. 1x; 24 V power supply unit, 36 W AC/DC (GlobTek, RR9LI1500LCPIMNAR6B). 1x; Power supply adapter cable. 1x; USB-C to USB-A cable, 1 m. |

| TECHNICAL DATA –PICK-UP |  |  |
| --- | --- | --- |
| Fastening of the pick-up via two M4 threaded holes (2). The stator coil is located in the yellow shaded area. The 41x38 mm² bottom of the housing is positioned at a distance of 1 to 10 mm from the rotor coil. Avoid metallic components in the immediate vicinity next to the rotor component and the coil, as this will reduce the reception quality. | POS.; DESCRIPTION. 1; SMA connector. 2; M4 threaded holes for mounting. 3; Stator coil. 4; Electric connector. PICK-UP. Distance pick-up / shaft; 1…10; mm. Material; plastic. Mounting; M4 screws. |
| TECHNICAL DATA – R+W APP |  |
| DATA TRANSFER. Frequency; 2,45; GHz. Bluetooth version; 5.0. Simultaneous connected IPKs; max. 4. Latency; < 150; ms. Synchronicity; ± 25; ms. Transmission power (max.); 4; dBm. Reception strength (min.); -90; dBm. Range1; ca. 10; m. 1; Specification valid for unobstructed view. With partial shielding, the range can be significantly reduced.. | DATA STORAGE. Record frequency; 1 - 500; Hz. Storage size; 512; Mb. Memory per measurand; 5,4; Mb/h. SYSTEM REQUIREMENTS. Operating system; min. Android 6.0. Bluetooth version; min. 4.0, better 5.0. Storage size for App; min. 30; Mb. Data export format; CSV-file. Synchronization; Unix Time Stamp. |

R+W Antriebselemente GmbH

Hattsteinstraße 4 · D-63939 Wörth am Main

Tel. +49 9372 9864-0 · Fax +49 9372 9864-20

info@rw-kupplungen.de · www.rw-couplings.com