IPK

INTELLIGENT PRECISION COUPLING

WITH INTEGRATED MEASUREMENT UNIT | BATTERY

|  | PROPERTIES |  |
| --- | --- | --- |
|  | ## FEATURES - Measurement of different dimensions - Measurement error < 1% (torque) - Integrated measurement amplifier - Display of the data on mobile device - Data export as CSV-file | ## SPEZIFICATIONS - Bluetooth 5.0 Low Energy - Magnetic charging plug - Sample rate up to 500 Hz - Recording rate up to 500 Hz - Speed resistance up to 8000 1/min |
|  | ## MEASURED DIMENSIONS - Torque - Rotation speed up to 2500 1/min - Acceleration - Optional: push / pull forces | ## DESIGN - Standard coupling with integrated sensors - Unchanged mechanical properties - Custom design on request |
|  | ## POWER SUPPLY VIA BATTERY - No wires necessary - Simple mounting - For mobile applications - High battery life |  |

| COMPATIBLE COUPLINGS |  |  |  |
| --- | --- | --- | --- |
| LINE-SHAFTS | DISC-PACK-COUPLINGS | SAFETY-COUPLINGS | STIFF COUPLINGS |
| ZA ¦ ZAE ¦ EZ2 | LP2 ¦ LP3 ¦ LP5 ¦ LPH | STF | Flange adapter |

| PROVISION OF MEASURED DATA |  |
| --- | --- |
| ## R+W GATEWAY - Connection on PC / SPS / Cloud solutions - USB-C Port (UART) - 8 analog outputs (-10 to +10 V) - Output of measured values - 8 digital outputs - Status of connection - Signal of occurred events - SMA connector for external antenna ## R+W APP - Display of all measured values - Min- / Max- and average values - Zeroing of values - Different types of diagrams - Detailed progress of measured data - Intuitive gesture control - Recording measurements for analytics e.g. in MS Excel - Data export as CSV-file |  |

| TECHNICAL DATA – INTELLIGENT PRECISION COUPLING |  |
| --- | --- |
| TORQUE MEASUREMENT. Resolution; 24; bit. Sample rate; 1 – 500; Hz. Error of indication; < 1; %. SPEED MEASUREMENT. Resolution; 16; bit. Sample rate; 1 – 500; Hz. Error of indication 0…660 1/min; 0,5; %. Error of indication 660…2500 1/min; 5; %. ACCELERATION MEASUREMENT. Number of axes; 3. Acceleration (max.); 30; g. Resolution; 16; bit. Sample rate; 1 – 500; Hz. OPTIONAL PUSH / PULL FORCE MEASUREMENT. Resolution; 24; bit. Sample rate; 1 – 500; Hz. Error of indication; < 2; %. OPERATION. Max. rotation speed; 8000; 1/min. Ambient conditions; normal. Temperature; -20 – 60 -4 – 140; °C °F. rel. centrifugal acceleration; 130; g. ESD; 4; kV. Protection class; IP 65. | POWER SUPPLY. Battery type; Li-Ion; Battery. Capacity; 320 / 3400; mAh. Charging voltage; 5; V. Charging connection; magnetic plug. Charging time; ca. 2 – 3; h. Power consumption1; 15; mA. Min. battery life1; 20; h. Max. battery life2; 150; Days. Standby current3; 0,1; mA. Idle mode current4; 5; µA. 1; With permanent 500 Hz sample rate and optional axial force measurement and smallest size. 2; With permanent 10 Hz sample rate and high battery capacity. 3; Visible in R+W App. 4; Not visible, activation per magnetic plug. INCLUDED ACCESSORIES. 1x; Magnetic charging cable. 1x; Calibration certificate. 1x; Operation manual. OPTIONAL ACCESSORIES. 1x; Magnetic charging cable. 1x; Gateway. |

| TECHNICAL DATA – R+W APP |  |
| --- | --- |
| DATA TRANSFER. Frequency; 2,45; GHz. Bluetooth version; 5.0. Simultaneous connected IPKs; 4. Latency; < 150; ms. Synchronicity; ± 25; ms. Transmission power (max.); 4; dBm. Reception strength (min.); -90; dBm. Range1; ca. 10; m. 1; Specification valid for unobstructed view. With partial shielding, the range can be significantly reduced.. | DATA STORAGE. Record frequency; 1 - 500; Hz. Storage size; 512; Mb. Memory per measurand; 5,4; Mb/h. SYSTEM REQUIREMENTS. Operating system; min. Android 6.0. Bluetooth version; min. 4.0, better 5.0. Storage size for App; min. 30; Mb. Data export format; CSV-file. Synchronization; Unix Time Stamp. |

| TECHNICAL DATA – GATEWAY (OPTIONAL) |  |
| --- | --- |
| The connections for supply and outputs are located on the top and bottom side of the gateway. | POS.; DESCRIPTION. 1; Digital outputs (do). 2; Digital outputs (do). 3; Power in (24 V) / power out (12 V). 4; Not used / debug. 5; Analog outputs (ao). 6; Analog outputs (ao). 7; External antenna connection, signal input. 8; Status LED. 9; USB-C port. 10; External antenna connection, signal output. 11; Fastening for mounting rail. |
| TECHNICAL DATA – GATEWAY (CONTINUATION) |  |
| ANALOG OUTPUTS. Amount; 8. Voltage range; ± 10; V. Resolution; 16; bit. Linearity deviation; < 1; %. Max. load; 20; mA. DIGITAL OUTPUTS. Amount; 8. Voltage; 24; V. Max. load; 0,5; A. SERIAL OUTPUT (UART). Amount; 1. Baud rate; Adjustable. Stop bits; 1. Parity; None. Data bits; 8. DATA TRANSFER. Frequency; 2,45; GHz. Bluetooth version; 5.0. Simultaneous connected IPKs; max. 4. Transmission rate1; up to 500; Hz. Latency; < 150; ms. Synchronicity; ± 25; ms. Transmission power (max.); 4; dBm. Reception strength (min.); -90; dBm. Range1; 10; m. 1; Specification valid for unobstructed view. With partial shielding, the range can be significantly reduced.. POWER SUPPLY. Supply voltage; 24; V. Current consumption (typ./max.); 50 / 100; mA. Power consumption (typ./max.); 1,2 / 2,4; W. OPERATION. Temperature; -20 – 60 -4 – 140; °C °F. Protection class; IP 20. Mounting rail; DIN EN 50022. | PIN ASSIGNMENT. PIN; POS. 1; POS. 2; POS. 3; POS. 5; POS. 6. 1; do5; do1; n.c.; GND; GND. 2; do6; do2; GND; ao8; ao4. 3; do7; do3; n.c.; ao7; ao3. 4; do8; do4; 24 V; ao6; ao2. 5; GND; GND; -; ao5; ao1. DIMENSION OF HOUSING. Weight: approx. 150 g. INCLUDED ACCESSORIES. 2x; Antenna (Laird, 001-0001). 1x; Connector, 4-pin (Amphenol, 20020006-G041B01LF). 4x; Connector, 5-pin (Amphenol, TJ0511530000G). OPTIONAL ACCESSORIES. 1x; 24 V power supply unit, 36 W AC/DC (GlobTek, RR9LI1500LCPIMNAR6B). 1x; Power supply adapter cable. 1x; USB-C to USB-A Kabel, 1 m. |

R+W Antriebselemente GmbH

Hattsteinstraße 4 · D-63939 Wörth am Main

Tel. +49 9372 9864-0 · Fax +49 9372 9864-20

info@rw-kupplungen.de · www.rw-couplings.com